# LEARNER GUIDE

Basic Urban Farming with Hydroponics

WSQ Course Code: TGS-2025053916
Version 2.0 · 29 August 2026
Tertiary Infotech Academy Pte Ltd · UEN 201200696W

# Document Version Control Record

| Version | Date | Description | Approved by |
| --- | --- | --- | --- |
| 1.0 | 24 March 2025 | Legacy approved course package baseline. | Course Development Team |
| 2.0 | 29 August 2026 | Substantial technical rebuild aligned to current Singapore context, farm tour and hydroponics-kit activity. | Dr Alfred Ang |

# Table of Contents

Course and Competency Alignment

How to Use This Guide

1. Hydroponic Root-Zone Control

2. Systems and Equipment

3. Nutrient and Water Management

4. Crop, Environment and Hygiene

5. Singapore Cases and Current Context

6. Aquaponics

Detailed Procedure A — Farm Tour Evidence Walk

Detailed Procedure B — Commission and Operate a Hydroponics Kit

Troubleshooting Decision Table

References and Current Sources

# Course and Competency Alignment

TSC: AGR-FOP-2002-1.1 Urban Farming Implementation and Management Control.

LO1: Perform farming activities and equipment operations according to standard operating procedures.

LO2: Execute farming operations in accordance with regulations, workplace safety and health, food-safety and housekeeping requirements.

Knowledge: equipment and machinery; farming SOPs; GAP/food safety/WSH; cleaning and maintenance.

Abilities: prepare; operate to targets; comply; clean, maintain and store equipment.

# How to Use This Guide

The slide deck explains mechanisms and decisions. This Learner Guide contains the detailed operating sequence. Always follow the site-specific SOP, product label, Safety Data Sheet and trainer instructions where they are more restrictive. Stop when a required control is absent.

# 1. Hydroponic Root-Zone Control

## Hydroponics replaces soil functions

Water carries dissolved ions; air spaces or aeration supply oxygen; channels/media support roots; monitoring replaces much of soil buffering.

## The operating variables

Record pH, EC, solution temperature, level/volume, flow and visible root condition. Interpret them together.

## Mass balance

Water uptake and evaporation can raise EC; selective uptake can change ion proportions even when EC looks stable.

## Plant demand

Light, heat, humidity, airflow, crop stage and canopy size change water and nutrient demand.

# 2. Systems and Equipment

## NFT

A shallow recirculating film provides water and oxygen but has low tolerance to pump interruption.

## DWC

Roots hang in aerated solution; air delivery and hygiene are critical.

## Kratky

A static reservoir develops an air gap; inappropriate refilling can remove that oxygen zone.

## Ebb-and-flow, drip and media

Irrigation duration, drainage and air-filled porosity must match the crop and media.

## Pump and drainage

Verify delivered flow at operating head, unobstructed returns, freeboard and power-off behaviour.

# 3. Nutrient and Water Management

## Source water

Baseline pH, EC and temperature; investigate alkalinity when pH continually rebounds.

## Nutrient mixing

Keep Part A and Part B concentrates separate; add each to water with circulation.

## pH and EC

Use crop-specific bands. EC is total ionic strength, not element-by-element analysis.

## Calibration

Use fresh standard, rinse correctly, record calibration and verify the instrument response.

## Correction

Add small measured amounts, circulate, remeasure, record before/action/after.

# 4. Crop, Environment and Hygiene

## Nursery and transplant

Accept only uniform, healthy plugs with established roots and intact media.

## Light and climate

Use PPFD/DLI, temperature, humidity/VPD and airflow trends to explain crop response.

## IPM

Prevent, monitor, identify, intervene lawfully, and verify. Sticky cards are trend tools.

## Root diagnosis

Inspect colour, odour, firmness, branching, flow, oxygen risk and chemistry before nutrient correction.

## Cleaning and maintenance

Remove residue before disinfection; follow concentration/contact time; rinse/dry; function-test before return to service.

# 5. Singapore Cases and Current Context

## Food Story 2

By 2035 Singapore targets local production capacity equivalent to 20% of fibre and 30% of protein needs. In 2025 output was about 8% and 25%, respectively.

## ComCrop

Rooftop hydroponics integrates sunlight, supplemental light, monitoring and manual crop operations.

## Sustenir

Controlled-environment vertical NFT demonstrates high area productivity and recirculation.

## Singrow

Cultivar development, controlled environment and automation target temperate crops in tropical conditions.

## Greenphyto

The 2026 multi-storey project demonstrates the scale-and-automation thesis; reported capacity must be separated from realised output.

# 6. Aquaponics

## Coupled biology

Fish, nitrifying microbes and plants share water and risk.

## Nitrogen conversion

Ammonia is converted to nitrite then nitrate; ammonia and nitrite are critical hazards.

## Oxygen and pH

Fish, roots and microbes all need oxygen; pH must balance animal health, nitrification and nutrient availability.

## Solids and biofilter

Remove solids before they foul roots; protect attached nitrifiers from disinfectants and abrupt changes.

## System choice

Aquaponics adds biological products and biological complexity; decoupling can separate optimisation at higher equipment and control cost.

# Detailed Procedure A — Farm Tour Evidence Walk

Attend the farm briefing and confirm permitted areas, photography rules and emergency instructions.

Why: The farm is a live workplace and may contain commercial information.

Acceptance evidence: Your checklist records the briefing and permission state.

Trace one water path from reservoir to delivery, root zone and return without touching equipment.

Why: Flow-path understanding reveals system boundaries and single points of failure.

Acceptance evidence: A labelled flow sketch with inlet, return, dosing/sample point and overflow.

Compare crop condition at inlet, middle and return positions and across tiers where present.

Why: Spatial patterns help distinguish shared-input from local flow/light issues.

Acceptance evidence: Three concise observation notes and permitted photographs where allowed.

Record how pH, EC, temperature and level/flow are measured and corrected.

Why: An operating control requires instrument, setpoint, frequency, action and verification.

Acceptance evidence: Completed water/nutrient control row with observation versus operator claim labelled.

Inspect WSH, food safety, pest monitoring, housekeeping and maintenance evidence.

Why: Safe compliant production depends on controls beyond plant growth.

Acceptance evidence: At least one existing control and one improvement opportunity, with no personal data.

Review the checklist with a peer, resolve unclear statements and submit the PDF evidence set.

Why: Peer review improves factual clarity and separates inference from observation.

Acceptance evidence: Signed checklist, flow sketch and only permitted photographs.

# Detailed Procedure B — Commission and Operate a Hydroponics Kit

Read the kit SOP; identify pump, reservoir, channel, tubing, return, planting sites and electrical isolation.

Why: Correct component identification prevents unsafe assembly and misrouting.

Acceptance evidence: Component check complete; no damaged or unknown parts.

Inspect for cleanliness, stability, cable damage, tubing kinks and safe separation of water and power.

Why: Visual acceptance prevents leaks, collapse and electrical exposure.

Acceptance evidence: Pre-use inspection signed; defects quarantined and reported.

Assemble only as shown in the SOP, fill with source water, keep electrical connectors dry and check for leaks before energising.

Why: Dry inspection and controlled filling limit spill and electrical consequence.

Acceptance evidence: Stable leak-free kit before power-on.

Energise the pump and confirm delivery to every planting site, unobstructed return and adequate reservoir freeboard.

Why: The farthest site and return behaviour establish functional flow.

Acceptance evidence: Flow verification recorded; no overflow or abnormal noise.

Measure and record source-water pH, EC and temperature using calibrated meters.

Why: Baseline separates source-water chemistry from nutrient addition.

Acceptance evidence: Time-stamped values with units, meter IDs and calibration status.

Add the trainer-specified Part A dose to water, circulate, then add Part B and circulate; never mix concentrates directly.

Why: Separation prevents concentrated calcium/phosphate/sulfate precipitation.

Acceptance evidence: No visible precipitate; dose and mixing times recorded.

Measure EC; make only small trainer-approved corrections with circulation between additions.

Why: Incremental correction reduces overshoot.

Acceptance evidence: EC is within the activity setpoint; before/action/after log complete.

Measure pH; if adjustment is authorised, use the labelled product, PPE and dedicated dosing tool, then circulate and remeasure.

Why: Acid/alkali dosing is a chemical task with a stop condition.

Acceptance evidence: pH is within setpoint and all additions are recorded.

Place an accepted seedling so its plug is supported, roots meet moisture and the crown remains stable.

Why: Correct placement supports root access without stem/crown damage.

Acceptance evidence: Seedling label, centred plug and intact healthy root evidence.

Conduct the trainer-controlled power-off/restart check only where spill and electrical controls are adequate.

Why: Commissioning must verify drain-down, reservoir capacity and restart recovery.

Acceptance evidence: No overflow/siphon issue; flow returns to every site after restart.

Record final values and obtain trainer verification before draining.

Why: The finished state must be demonstrably controlled.

Acceptance evidence: Final pH/EC/temperature/flow record and trainer check.

Isolate power, drain through the approved route, remove residue, clean, rinse, dry, reassemble and store.

Why: Cleaning and maintenance prevent contamination and preserve equipment readiness.

Acceptance evidence: Dry complete kit, chemicals secured, maintenance/defect log completed.

# Troubleshooting Decision Table

| Observation | Check first | Do not assume | Escalation/acceptance |
| --- | --- | --- | --- |
| Wilting with full reservoir | Flow, root condition, solution temperature, aeration | That more water or fertiliser is automatically needed | Restore flow/oxygen safely; isolate disease risk; record recovery |
| EC rises as level falls | Water loss, top-up history, instrument verification | That the nutrient recipe is too strong | Follow SOP top-up/correction and verify after mixing |
| pH repeatedly rises | Source alkalinity, new media, uptake pattern | That one large acid dose is safe | Use incremental correction; escalate persistent rebound |
| Brown slimy roots | Temperature, DO/aeration, flow, odour, shared loop | That leaf feeding solves the root cause | Protect reservoir, isolate per SOP, sanitise only as approved |
| Uneven plants along channel | Delivered flow, slope, blockage, light uniformity | That seed genetics is the only cause | Correct physical non-uniformity and verify endpoint flow |

# References and Current Sources

https://www.tertiarycourses.com.sg/wsq-basic-urban-farming-with-hydroponics.html

https://www.sfa.gov.sg/news-publications/newsroom/singapore-food-statistics-2025

https://www.mse.gov.sg/latest-news/oral-reply-to-parliamentary-question-on-revised-local-production-targets/

https://www.sfa.gov.sg/food-science-and-technology/technology/agriculture-technologies

https://www.sfa.gov.sg/fromSGtoSG/farms/farm/Detail/comcrop

https://www.sfa.gov.sg/fromSGtoSG/farms/farm/Detail/sustenir-agriculture

https://www.sfa.gov.sg/food-for-thought/article/detail/hydroponics-getting-to-the-root-of-the-myths

https://www.sfa.gov.sg/recognition-programmes-grants/grants/agri-food-cluster-transformation-act-fund

https://enterprise.nus.edu.sg/startup-story/singrow-2/

https://www.businesstimes.com.sg/singapore/smes/singapore-food-output-gets-s80-million-boost-opening-greenphytos-vertical-farm-tallest-world

https://www.rhs.org.uk/vegetables/hydroponics

https://extension.illinois.edu/blogs/good-growing/2026-01-16-growing-without-soil-getting-started-hydroponics-home

https://extension.unr.edu/publication.aspx?PubID=2756